OPERATIONAL PROCEDURE

Safety and PPE Program

Mobile Home Park Maintenance

Parakeet Communities  ·  Field Operations SOP

Process Owner: Regional Manager

Triggers downstream: SOP 108 — Incident Report (activated upon any PPE-related injury or safety incident)

SOP Location: SharePoint > Training SOPs

PURPOSE

This SOP establishes Parakeet Communities’ internal PPE program for all field staff, aligned with OSHA standards. Its purpose is to protect maintenance technicians and other field staff from injuries, illnesses, and workplace hazards while performing duties throughout the community. PPE is the last line of defense and is always used in conjunction with proper training, safe work practices, and hazard controls.

SCOPE

Applies to: All Parakeet Communities field staff performing work that involves a recognized hazard. Contractors and vendors operating on Parakeet properties are responsible for their own PPE compliance and are excluded from this program.

Trigger: Any time a field staff member begins work on-site that involves a recognized hazard as defined in this SOP.

Finish line: Work task is completed and all PPE has been removed, inspected post-use, and properly stored.

TIMELINE 1: PPE SELECTION & REQUIREMENTS

Roles and Responsibilities

- Maintenance Technician — Identifies hazards and selects appropriate PPE before each task.

- Maintenance Manager — Confirms PPE selection for non-routine or high-risk tasks; enforces daily compliance.

- Community Manager — Conducts informal spot-checks and reports non-compliance to the Regional Manager.

- Regional Manager — Process owner; oversight and accountability for the entire program.

- Director of Operations — Program-level accountability; escalation path for systemic non-compliance.

PPE Requirements by Hazard

| PPE Type | When Required | Approved Equipment | Key Hazards |
| --- | --- | --- | --- |
| Eye Protection | Mowing/trimming, power tools, cutting pipe, grinding metal, chemicals, pressure washing, demolition | ANSI Z87.1 safety glasses; safety goggles; face shield (in addition to safety glasses) | Flying debris, metal fragments, chemical splashes, dust |
| Hand — Nitrile Gloves | General maintenance, painting, chemical handling, waste cleanup | Nitrile gloves | Cuts, punctures, chemical burns |
| Hand — Cut-Resistant Gloves | Sheet metal, broken glass, utility knife, appliance repairs | Cut-resistant gloves | Cuts, punctures |
| Hand — Chemical-Resistant Gloves | Drain cleaners, herbicides, cleaning chemicals | Chemical-resistant gloves | Chemical burns |
| Hand — Electrical Gloves | Electrical troubleshooting, working near energized equipment (breaker changes) | Voltage-rated insulating gloves | Electrical shock |
| Foot Protection | All field work. Waterproof boots required for: water/sewer repairs, storm cleanup, pressure washing | ASTM-rated safety toe boots, slip-resistant soles; waterproof boots (situational) | Falling objects, slips/falls, sharp debris, wet conditions |
| Head Protection | Tree work, excavation, equipment operation, roofing, any overhead hazards | Hard hat | Falling branches/tools, low clearances, electrical exposure |
| Hearing Protection | Lawn equipment, chainsaws, generators, air compressors | Ear plugs or ear muffs | Noise-induced hearing loss |
| Respiratory — Dust Mask / N95 | Drywall sanding, dusty units, insulation work | N95 / dust mask | Dust, particulates |
| Respiratory — Half-Face Respirator | Mold remediation, chemical applications, painting, strong fumes | Half-face respirator | Chemical fumes, mold spores |
| Respiratory — Full-Face Respirator | Severe mold conditions, hazardous chemical exposure | Full-face respirator | Hazardous chemical exposure |

Tasks

1.   Maintenance Technician identifies the hazard(s) present for the upcoming task using the PPE Requirements by Hazard matrix above.

2.   Maintenance Technician selects the appropriate PPE for all identified hazards before beginning any work.

3.   Maintenance Manager confirms correct PPE selection for any non-routine or high-risk task prior to work beginning.

⚠  IMPORTANT: Most common compliance gaps: safety glasses during mowing, long pants in the field, and hearing protection around lawn equipment. Supervisors must reinforce these during routine walkthroughs.

TIMELINE 2: WEEKLY PPE INSPECTION & READINESS

Roles and Responsibilities

- Maintenance Technician — Completes the Weekly PPE Inspection Checklist at the start of each work week and submits it via FastField Forms.

- Maintenance Manager — Reviews submitted checklists; ensures replacement PPE is available on-site.

- Regional Manager — Reviews PPE inventory status during property visits.

Tasks

1.   Maintenance Technician completes the Weekly PPE Inspection Checklist in FastField Forms at the start of each work week.

Notes: Form name: Weekly PPE Inspection Checklist. See reproduced checklist in the Forms & Checklists section of this SOP.

2.   Maintenance Technician removes and sets aside any PPE that is damaged, worn, or defective. That item must not be used until it is replaced.

3.   Maintenance Technician submits a replacement request to their Maintenance Manager for any PPE flagged for replacement.

4.   Maintenance Manager reviews the submitted checklist in FastField Forms and ensures all replacement PPE is available on-site before the next work shift.

TIMELINE 3: TASK-BASED PPE PROTOCOLS

Roles and Responsibilities

- Maintenance Technician — Dons required PPE per task type before beginning work; adds situational PPE as conditions require.

- Community Manager / Maintenance Manager — Conducts informal PPE compliance checks during active work tasks.

PPE Requirements by Task

| Task Type | Required PPE | Additional (Situational) | Key Hazards |
| --- | --- | --- | --- |
| Plumbing Repairs | Safety glasses, nitrile gloves, waterproof boots | Face shield, disposable coveralls, respirator (if sewage exposure present) | Sewage contamination, biological hazards, pressurized water |
| Electrical — Breaker Changes Only | Safety glasses, voltage-rated gloves | Non-contact voltage tester | Electrical shock. All major electrical work performed by licensed vendor. |
| Groundskeeping | Safety glasses, hearing protection, high-visibility vest, work gloves, safety boots | Face shield, dust mask | Flying debris, heat stress, equipment injuries |
| Demolition & Unit Turns | Safety glasses, gloves, steel-toe boots | Respirator, disposable coveralls, hard hat | Dust, mold, nails, falling debris |

Tasks

1.   Maintenance Technician dons all required PPE listed in the Task Requirements table above before beginning any work.

2.   Maintenance Technician adds any additional situational PPE required for specific site conditions (e.g., a respirator if sewage exposure is present during plumbing work).

3.   Community Manager or Maintenance Manager conducts informal observation of PPE compliance during active work and addresses any gaps immediately.

⚠  IMPORTANT: Non-compliance with PPE requirements is documented by the Community Manager and reported to the Regional Manager. Disciplinary action follows the standard performance improvement process.

clear.

TIMELINE 4: HEAT STRESS MANAGEMENT

Roles and Responsibilities

- Maintenance Manager / Community Manager — Monitors heat index before outdoor work begins; ensures heat safety supplies are stocked and available.

- Maintenance Technician — Monitors themselves and co-workers for signs of heat illness; reports symptoms immediately and stops work.

Tasks

1.   Maintenance Manager or Community Manager checks the heat index before outdoor work begins on any day forecasted at or above 90°F.

2.   Maintenance Manager ensures cooling towels, water stations, electrolyte packets, and shaded rest areas are in place before outdoor work begins on heat-alert days.

Notes: Supplies must be available at the start of the shift — not sourced mid-day.

3.   Maintenance Technician monitors themselves and co-workers throughout outdoor tasks for signs of heat exhaustion: heavy sweating, dizziness, headache, muscle cramps, or nausea.

4.   Maintenance Technician reports any heat exhaustion symptoms to the Maintenance Manager or Community Manager immediately and stops outdoor work.

5.   Community Manager or Maintenance Manager removes the affected staff member from heat exposure, provides water and cooling, and monitors closely for escalation to heat stroke.

⚠  IMPORTANT: Heat stroke is a MEDICAL EMERGENCY. Signs: confusion, loss of consciousness, hot dry skin, or body temperature above 103°F. CALL 911 IMMEDIATELY — do not wait. Notify the Regional Manager as soon as possible after calling 911.

→  HANDOFF: Any workplace injury or medical emergency resulting from heat illness triggers SOP 108 — Incident Report.

TIMELINE 5: PPE MAINTENANCE, STORAGE & REPLACEMENT

Roles and Responsibilities

- Maintenance Technician — Cleans, inspects, and properly stores PPE after each use; submits replacement requests to their supervisor.

- Maintenance Manager — Conducts monthly PPE inventory check; submits replacement orders to the Regional Manager.

- Regional Manager — Reviews PPE inventory during property visits; approves replacement orders.

Tasks

1.   Maintenance Technician cleans and properly stores all PPE after each use per manufacturer guidelines.

2.   Maintenance Technician inspects PPE for wear or damage after each use and flags any item needing replacement.

3.   Maintenance Technician submits a PPE replacement request to their Maintenance Manager for any item that cannot continue to be used safely.

4.   Maintenance Manager conducts a full PPE inventory check monthly and reviews status with the Regional Manager during each property visit.

5.   Regional Manager reviews PPE inventory status during property visits and approves replacement orders.

Notes: Replacement requests flow from Maintenance Technician → Maintenance Manager → Regional Manager for approval and ordering.

FORMS & CHECKLISTS

Weekly PPE Inspection Checklist

Status: Reproduced below. Also available as a standalone form in FastField Forms.

Filled out by: Maintenance Technician — completed at the start of each work week.

Submitted to: FastField Forms (logged). Visible to Maintenance Manager and Regional Manager.

| ☐ | Inspection Item |
| --- | --- |
| ☐ | Safety glasses are clean and free of scratches or damage |
| ☐ | All gloves are intact and free from holes or tears |
| ☐ | Safety boots are in good condition (soles not worn, no cracks) |
| ☐ | Hard hat is not cracked or dented |
| ☐ | Harness and lanyard have been inspected and are functioning properly |
| ☐ | Respirator is clean and functioning; filters/cartridges not expired |
| ☐ | Hearing protection (ear plugs or ear muffs) is available and undamaged |
| ☐ | High-visibility vest is available and in good condition |
| ☐ | First aid kit is stocked and up to date |
| ☐ | Water and hydration supplies (cooling towels, electrolyte packets) are stocked |
| Completed By: _________________________________________ Date: _____________________ Property: _______________________________________ Submitted via FastField Forms |  |

RELATED RESOURCES

- FastField Forms — Weekly PPE Inspection Checklist (form to be created; name: “Weekly PPE Inspection Checklist”)

- SOP 108 — Incident Report (SharePoint > Training SOPs)

- SharePoint > Training SOPs — Location of this SOP and all related field operations SOPs

FREQUENTLY ASKED QUESTIONS

Q: What do I do if my PPE is damaged or runs out mid-shift?

Stop the task requiring that PPE immediately. Notify your supervisor and do not resume the task until proper, functional PPE is available.

Q: Who is responsible for providing PPE — do I have to buy my own?

Parakeet provides all required PPE. If you need a replacement or additional equipment, submit the request to your supervisor.

Q: What do I do if I see a co-worker not wearing required PPE?

Remind them of the requirement directly. If they continue working without required PPE, report the situation to the Community Manager, who will document and escalate to the Regional Manager.

NEW HIRE TRAINING & ACKNOWLEDGMENT

All new maintenance staff must review this SOP during onboarding. A signed acknowledgment confirming they have read, understood, and agree to comply with this program must be placed in their maintenance binder. The Maintenance Manager is responsible for administering the onboarding review and filing the signed acknowledgment.

REVIEW & REVISION

This SOP is reviewed annually, or following any significant PPE-related incident, regulatory change, or process update.

| Date | Revised By | Changes Made |
| --- | --- | --- |
| August 1, 2026 | Natalie | Initial document created. |