parakeet communities

Signage Package

Sign Specifications  ·  Installation Standards  ·  Hardscape Guide

For Vendors, Contractors & Internal Team

Section 1 — Sign Specifications

All signs are designed and fabricated by Elements, Inc. The drawings below are the approved specifications for each sign type used across Parakeet Communities properties.

Sign Type 1 — Main Entry Monument Sign

Main Entry Monument Sign — 79"W × 72"H face, 32" above grade, 24" below grade

Sign Type 2 — Community Office Sign

Community Office Sign — 32"W × 32"H, 30" above grade, 24" below grade

Sign Type 3 — Private Property Sign

Private Property Sign — 22"W × 22"H, 30" above grade, 24" below grade

Sign Type 4 — Speed Limit Sign

Speed Limit Sign — 12"W × 18"H, 40" above grade, 24" below grade

Sign Type 5 — Street / Wayfinding Blade Sign

Street Blade Sign — 38"W × 6"H blade, 96" above grade, 24" below grade. Optional 48" wood insert.

Section 2 — Hardscape Specification

A raised planter surround is required at every main entry monument sign. Select either the Gray or Taupe/Beige color scheme to complement the community's existing aesthetics.

| Gray Stone (Option A) | Taupe/Beige Stone (Option B) |
| --- | --- |

Overall Planter Dimensions

| Item | Measurement |
| --- | --- |
| Outside Length | 9'-0" (108") |
| Outside Width | 4'-6" (54") |
| Finished Wall Height | 12" (2 courses) |
| Wall Thickness | 8" |
| Interior Stone Area | ≈ 44 sq ft |

Center the wall beneath the sign on all sides. Adjust field dimensions as needed.

Retaining Wall Block

Style: Split-face concrete retaining wall block, 4"H × 8"W × 12"D approx.

Color: Gray or Taupe/Beige to match selected color scheme.

Approved products: Keystone Compac, Belgard Diamond Pro Mini, Pavestone Retaining Wall Block, Oldcastle Mini Beltis.

| Item | Quantity |
| --- | --- |
| Perimeter | ≈ 27 linear feet |
| Courses | 2 |
| Block Quantity | 60–65 blocks (includes 10% waste) |

Base Preparation

Excavate approximately 10 inches below finished grade.

Install geotextile fabric.

Place 6 inches of compacted #57 stone.

Finish with 1 inch of leveling screenings.

Compact every 2 inches during fill.

Wall Installation

First course: Set completely level. Embed approximately 1" below surrounding grass grade on compacted stone base.

Second course: Running bond pattern — vertical joints must fall at center of block below.

Adhesive between courses: Loctite PL Premium, SRW Vertical Instant Lock, or Sakrete Landscape Adhesive.

Interior Fill

Install landscape fabric over compacted soil.

Fill with 1½" washed river rock matching wall color. Target depth: 4".

Keep rock ≈ 1" below top of wall for a clean finished look.

Place #57 drainage stone (6" wide, full wall height) behind inside face of wall.

Section 3 — Material Summary

Quantities below are per sign installation and include waste allowance. Verify against field measurements before ordering.

| Material | Approx. Quantity |
| --- | --- |
| Retaining Wall Block | 60–65 blocks |
| #57 Base Stone | 1.2 cubic yards |
| Decorative River Rock (Gray or Taupe/Beige) | 0.5 cu yd (≈ 900–1,000 lbs) |
| Geotextile Fabric | 60 sq ft |
| Landscape Block Adhesive | 4 tubes |
| Leveling Screenings | 0.15 cubic yard |
| #57 Drainage Stone (behind wall) | 0.20 cubic yard |

Construction Notes

Center the planter perfectly beneath the monument sign.

Finished wall height: 12" (2 courses).

Maintain minimum 1% slope away from posts to prevent standing water.

Use running bond pattern — stagger vertical joints every course.

Before digging: verify no irrigation lines or electrical conduits are in the excavation area.

Florida sandy soils: extend stone base 6" beyond wall footprint on all sides for stability.

Section 4 — Landscaping & Visual Interest

One landscaping option is required at every main entry sign. Select the option that best fits the community's maintenance capacity and aesthetic goals.

Option 1 — Permanent Plants ¦ Low Maintenance

Drought-tolerant, zone-specific species growing no taller than 24". Plant 2–4 specimens in the stone field, spaced equally in front of the sign posts. Use the post spacing as a centering guide. Occasional trimming required.

Option 1 — Ornamental grasses planted in stone field (Southgate reference)