1. Permits & Coordination

- Make sure zoning is cleared for the home.

- Pull all required permits – building, electrical, plumbing, septic, mechanical.

- Confirm setbacks, orientation, and driveway access against the site plan.

- Coordinate with utility companies for hookups.

- Double-check flood and wind zone requirements are covered.

2. Site Clearing and Grading

- Clear the lot – trees, brush, stumps, debris, old pads, or abandoned utilities.

- Grade the site to drain away from the home (5% slope within 10 ft).

- No ponding or standing water left behind.

3. Foundation & Pad Prep

- Confirm with county/city what foundation type they require (slab, footers, piers, or compacted dirt base).

- Remove/replace any cracked or non-compliant concrete pad.

- Install foundation system per code and manufacturer specs.

- Verify pad dimensions match the home.

- Vapor barrier installed if not going on slab.

- Lay out and check pier spacing against HUD/FL code and manufacturer’s diagram.

4. Utility Infrastructure

- Water line in, with shutoff within 3 ft of the home.

- Sewer/septic lateral in, cleanout within 5 ft of connection.

- Electrical pedestal (200-amp unless code calls otherwise) installed in correct spot.

- The grounding system meets code.

5. Pre-Delivery Prep

- Stake corners, confirm orientation matches plan.

- Mark anchor locations before delivery.

- Take photos of lot prep for records.

6. Home Delivery & Placement

- Direct trucks onto the lot and ensure safe placement.

- Set home sections on the pad/foundation.

- Align and level home to manufacturer’s spec.

- Seal marriage line (double wide)

7. Anchoring & Tie-Down

- Install tie-downs

- Include longitudinal stabilizers, vertical ties, or over-the-top ties if required.

- Torque test all anchors and log results.

- Block under all required load points.

8. Hookups & Finishing

- Connect and test water.

- Connect and test sewer/septic.

- Hook up electric, schedule inspection.

- HVAC/ Heat Pump installed and running.

- Skirting installed within 60 days, vented, and secure.

9. Final Site Work & Inspection

- Backfill/landscape

- Driveway and walkway in per site plan.

- Interior trim-out and finish mechanical hookups complete.

- Test smoke detectors, GFCIs, HVAC, and plumbing.

- Schedule and pass final county/municipal inspection.

- CO (Certificate of Occupancy) in hand.

10. Close-Out

- Provide warranty info and inspection reports

- Lot cleaned up and free of debris.