Permits & Approvals

- Make sure zoning is cleared for the home.

- Pull all required permits —GC should have all necessary permits pulled (building, electrical, plumbing, mechanical, septic)

- Double-check HUD standards.

- Confirm flood zone and wind zone requirements are covered.

Site Prep

- Verify lot boundaries and setbacks.

- Clear trees, brush, and debris — lot needs to be clean and graded right.

- Remove, add, or replace pad if needed.

- Confirm with county/city/contractor what foundation they’re calling for (slab, footers, dirt, columns).

- Compact soil, set slope so water drains away.

- No standing water left behind.

Utilities & Infrastructure

- Verify locations/depths for water, sewer, and electric.

- Install sewer lateral with cleanout within 5 ft if needed.

- Water line needs shutoff within 3 ft of connection.

- Install 200-amp electrical pedestal (or whatever code requires).

- Grounding meets NEC/FL code.

Foundation & Support

- Prep pad (stone, slab, or pier system as approved).

- Double-check pad size matches manufacturer’s instructions.

- Soil test if the county requires it.

- Vapor barrier in place if no slab.

- Piers spaced by HUD/FL code and manufacturer’s diagram.

Delivery & Placement

- Confirm trucks can access — clearance, turning radius, stable roads.

- Stake lot corners, confirm orientation.

- Mark anchor locations. (GC)

Install & Tie-Down

- Set home per site plan.

- Block on piers at required load points.

- Tie-downs installed per code.

  - Longitudinal stabilizers

  - Over-the-top ties (if required)

  - Anchors set to depth/torque

- Torque test anchors.

- Seal marriage line.

Utility Hookups

- Connect and test water.

- Connect and test sewer/septic.

- Connect electric, schedule inspection.

- Hook up gas (if needed), test.

- Skirting installed, vented, secure (must be done within 60 days).

Final Punch & Inspections

- Trim-out and finish mechanicals.

- Test smoke detectors, GFCI outlets, HVAC, plumbing.

- Backfill/landscape to stop erosion.

- Install driveway/walkway.

- Call for final inspection.

- Get Certificate of Occupancy (CO).

Marketing & Lease-Up/Sales

- Home staged/cleaned (inside and out).

- Photos taken for listings.

- Signage up (For Sale/Lease, branding, contact info).

- Rent Manager updated — pricing, status, availability.

- Listings live (MH Village, Zillow, Craigslist, Facebook, referrals, etc.).

- Sales team ready for tours, apps, closings.